# Public Comment

# G 99 – 12

Individual Consideration Agenda

This item is on the agenda for individual consideration because a public comment was submitted.

Public Comment:

Name: Jane Malone, National Center for Healthy Housing (jmalone@nchh.org)

Modify the proposal as follows:

Add new text as follows:

SECTION 425

RADON REDUCING CONSTRUCTION FEATURES FOR GROUP E OCCUPANCIES.

425.1. General. Group E Occupancies shall comply with the provisions of this section if the building is located in an area of High (Zone 1) Radon Potential as determined by Figure 425-2 and Table 425-1.

425.2. Radon Reducing Construction Features. Buildings, including occupiable space directly above a garage, shall be equipped with radon reducing features in accordance with Sections 425.2.1 through 425.2.8.

425.2.1. Gravel. A uniform layer of clean aggregate that meets the specifications of Size Number 4, 5, 56, or 6 of ASTM C33 shall be installed below the foundation slab. The depth of gravel shall equal or exceed the diameter of the pipe installed according to Section 425.2.3.

425.2.2. Vapor Barrier. A continuous vapor barrier meeting ASTM E1745 Class A, B or C shall be installed under the slab in basement and slab-on-grade construction, and on the soil in crawl space construction. Any seams of the vapor barrier shall be overlapped not less than 12 inches (305 mm) and sealed with a caulk complying with ASTM C920 Class 25 or higher or equivalent method.

425.2.3 Vent Stack. A solid, rigid, gas tight, non-perforated, ABS or PVC vent pipe vent pipe shall be installed in a continuous vertical stack from the "T" pipe fitting for each suction point installed according to Section 425.2.3 through the interior of the building to the termination point installed according to Section 425.2.6. The vent stack pipe shall be installed such that water will not collect. Figure 425-1 illustrates the vent stack.

425.2.3.1 Pipe Dimensions. Mitigation system pipe shall be not less than 4 inches (102 mm) nominal inside diameter. Pipe wall thickness shall be Schedule 40.

425.2.3.2 Coverage Area. One independent vent stack shall be installed for every contiguous 5,000 square feet (464 sq. m.) of slab or crawlspace area.

Exception: If 6 inch (152 mm) nominal inside diameter pipe is used, there may be one independent vent stack for every contiguous 15,000 square feet (1,392 sq. m.) of slab or crawlspace area.

425.2.3.3 Interconnected Coverage Areas. Piping to interconnect the pipes mitigating separate areas in combination basement/crawl space buildings, combination slab-on-grade/crawl space buildings, and slabs divided by internal footings that connect two of these areas to the same vent stack shall cover up to a total area of 5,000 square feet (464 sq. m).

Exception: If 6 inch (152 mm) nominal inside diameter pipe is used, the piping shall cover up to a total area of 15,000 square feet (1,392 sq. m.) of slab or crawlspace area.

425.2.3.4 Suction Points (Basement Slab and Slab on Grade Foundations). A "T" pipe fitting or equivalent shall be installed for each coverage area in the sub-slab aggregate. Each of the horizontal openings of the "T" pipe fitting shall be connected to a minimum of 10 feet (3 m) of perforated pipe with a minimum of 1 sq. inch (645 sq. mm) of opening per lineal foot of pipe.  The perforated pipe shall be completely covered by the sub-slab aggregate. The vertical portion of the "T" pipe fitting shall be connected to an independent solid vent stack.

425.2.3.5. Suction Points (Crawl Spaces). Crawl spaces shall be provided with a "T" pipe fitting or equivalent for each coverage area. Each of the horizontal openings of the "T" pipe fitting shall be connected to a minimum of 10 feet (3 m) of perforated pipe with a minimum of 1 sq. inch (645 sq. mm) of opening per lineal foot of pipe and installed upon the soil. The vertical portion of the "T" pipe fitting shall be connected to an independent solid vent stack.

425.2.3.6. Suction Points (Sump Cover). A sump cover shall not be used as a vent pipe location.

425.2.3.7 Discharge Point. Each independent solid vent stack pipe shall terminate at an approved location, as prescribed in Section 425.2.6, on the exterior of the building.

425.2.4. Sealing. Openings and penetrations shall be sealed according to Sections 425.2.4.1 through 452.2.4.5.

425.2.4.1. Foundation Walls and Floors. Joints, openings and penetrations in foundation walls and floors, in ground contact shall be sealed by utilizing a caulk complying with ASTM C920 Class 25 or higher, in order to close off the radon entry routes. Prior to sealing, backer rods or equivalent shall be used to fill gaps greater than one inch.

425.2.4.1.1. Hollow masonry unit walls. The top course of hollow block masonry foundation walls shall be made of solid masonry units or the top course shall be fully grouted. The top course under the full width of door and window openings shall be made of solid masonry units or the hollow masonry units shall be fully grouted. Where a brick veneer or other masonry ledge is installed, the course immediately below the ledge shall be made of solid masonry units or the top course shall be fully grouted. Other penetrations through walls shall be sealed.

425.2.4.2. Floor drains. Floor drains and condensate drains shall not allow radon entry

425.2.4.3. Sump Cover. A sump cover which closes off the radon entry routes shall be provided for all sump installations.

425.2.4.4. Ductwork. Where ductwork passes through a crawl space, or through or beneath a slab, all openings and joints shall be seamless or properly taped or sealed water-tight.

425.2.4.5. Vapor Barrier (Crawl Space). The vapor barrier in a crawl space shall turn up onto the foundation walls not less than 12 inches (153 mm) and shall be continuously sealed to the wall with a caulk complying with ASTM C920 Class 25 or higher or equivalent method. Openings or penetrations in the barrier shall be sealed.

425.2.5. Provision for Depressurization Fan. A section of the vent stack pipe that is located outdoors or in a non-conditioned space above the basement or crawl space shall be accessible for the future installation of an in-line fan. The fan shall not be mounted in any location where pipe positively pressured by the fan is located inside conditioned or occupiable space.

425.2.5.1. Accessible Fan Installation Location. A space having a vertical height of not less than 48 inches [122 cm] and a diameter of not less than 21 inches [53 cm] shall be provided in the area designated for a depressurization fan. The vent stack pipe shall be centered in this space.

425.2.5.2. Electrical Connection for Fan. A branch circuit shall supply an outlet box installed within 6 feet (1.8 m) of the area designated for a depressurization fan.

425.2.6. Vent Stack Termination. The vent stack pipe shall discharge outdoors.

425.2.6.1.	Rooftop Termination. Vent stack pipes shall terminate at least 2 feet (610 mm) above the roof surface, measured from the highest point where the pipe intersects the roof. When a vent stack pipe terminates on an occupiable roof, the pipe shall extend at least 10 feet (3 m) above the roof surface.

Exception: In a building more than three stories in height, the vent stack pipe may terminate through an exterior wall provided that the termination point is at least 20 feet (6 m) above grade and is at least 10 feet (3 m) in any direction from any operable window, door, or other gravity intake opening into the building.

425.2.6.2. Clearance from Other Buildings and Lots. No vent terminal shall be closer than 25 feet (7.6 m) horizontally from any adjacent building or lot line.

425.2.7. Labeling. Radon vent pipes shall be identifiable and clearly labeled as a radon reduction system at intervals of at least every 10 feet (3 m) and at least once in every room or space. The radon reduction system label of any section of vent pipe above the roof shall caution against placement of air intake openings within 10 feet (3 m) of the vent pipe discharge.

425.2.8. Air Passages. Air passages that penetrate the conditioned envelope of the building or openings installed in top-floor ceilings shall be closed, gasketed or otherwise sealed with materials approved for such applications.

Add new standard to Chapter 35 as follows:

ASTM  E 1745-11	Standard Specification for Plastic Water Vapor Retarders Used in Contact with Soil or Granular Fill under Concrete Slabs

Figure 425-2 EPA Map of Radon Zones

The United States Environmental Protection Agency and the United States Geological Survey have evaluated the radon potential in the United States and developed this map of radon zones.  The map assigns each of the 3,141 counties in the United States to one of three zones based on radon potential.  Each zone designation reflects the average short-term radon measurement that can be expected in a building without radon mitigation. The zone with potential for the highest level of radon is Zone 1. Table 425.1 lists the Zone 1 counties illustrated on the map.

Table 425-1

List of Counties with High Radon Potential (Zone 1)

| ALABAMA | CONNECTICUT | Morgan | Wabash | Thomas | Cass | Washington |
| --- | --- | --- | --- | --- | --- | --- |
| Calhoun | Fairfield | Moultrie | Warren | Trego | Hillsdale | Watonwan |
| Clay | Middlesex | Ogle | Washington | Wallace | Jackson | Wilkin |
| Cleburne | New Haven | Peoria | Wayne | Washington | Kalamazoo | Winona |
| Colbert | New London | Piatt | Wells | Wichita | Lenawee | Wright |
| Coosa |  | Pike | White | Wyandotte | St. Joseph | Yellow Medicine |
| Franklin | GEORGIA | Putnam | Whitley |  | Washtenaw |  |
| Jackson | Cobb | Rock Island |  | KENTUCKY |  | MISSOURI |
| Lauderdale | De Kalb | Sangamon | IOWA | Adair | MINNESOTA | Andrew |
| Lawrence | Fulton | Schuyler | All Counties | Allen | Becker | Atchison |
| Limestone | Gwinnett | Scott |  | Barren | Big Stone | Buchanan |
| Madison |  | Stark | KANSAS | Bourbon | Blue Earth | Cass |
| Morgan | IDAHO | Stephenson | Atchison | Boyle | Brown | Clay |
| Talladega | Benewah | Tazewell | Barton | Bullitt | Carver | Clinton |
|  | Blaine | Vermilion | Brown | Casey | Chippewa | Holt |
| CALIFORNIA | Boise | Warren | Cheyenne | Clark | Clay | Iron |
| Santa Barbara | Bonner | Whiteside | Clay | Cumberland | Cottonwood | Jackson |
| Ventura | Boundary | Winnebago | Cloud | Fayette | Dakota | Nodaway |
|  | Butte | Woodford | Decatur | Franklin | Dodge | Platte |
| COLORADO | Camas |  | Dickinson | Green | Douglas |  |
| Adams | Clark | INDIANA | Douglas | Harrison | Faribault | MONTANA |
| Arapahoe | Clearwater | Adams | Ellis | Hart | Fillmore | Beaverhead |
| Baca | Custer | Allen | Ellsworth | Jefferson | Freeborn | Big Horn |
| Bent | Elmore | Bartholomew | Finney | Jessamine | Goodhue | Blaine |
| Boulder | Fremont | Benton | Ford | Lincoln | Grant | Broadwater |
| Chaffee | Gooding | Blackford | Geary | Marion | Hennepin | Carbon |
| Cheyenne | Idaho | Boone | Gove | Mercer | Houston | Carter |
| Clear Creek | Kootenai | Carroll | Graham | Metcalfe | Hubbard | Cascade |
| Crowley | Latah | Cass | Grant | Monroe | Jackson | Chouteau |
| Custer | Lemhi | Clark | Gray | Nelson | Kanabec | Custer |
| Delta | Shoshone | Clinton | Greeley | Pendleton | Kandiyohi | Daniels |
| Denver | Valley | De Kalb | Hamilton | Pulaski | Kittson | Dawson |
| Dolores |  | Decatur | Haskell | Robertson | Lac Qui Parle | Deer Lodge |
| Douglas | ILLINOIS | Delaware | Hodgeman | Russell | Le Sueur | Fallon |
| El Paso | Adams | Elkhart | Jackson | Scott | Lincoln | Fergus |
| Elbert | Boone | Fayette | Jewell | Taylor | Lyon | Flathead |
| Fremont | Brown | Fountain | Johnson | Warren | Mahnomen | Gallatin |
| Garfield | Bueau | Fulton | Keary | Woodford | Marshall | Garfield |
| Gilpin | Calhoun | Grant | Kingman |  | Martin | Glacier |
| Grand | Carroll | Hamilton | Kiowa | MAINE | McLeod | Granite |
| Gunnison | Cass | Hancock | Lane | Androscoggin | Meeker | Hill |
| Huerfano | Champaign | Harrison | Leavenworth | Aroostook | Mower | Jefferson |
| Jackson | Coles | Hendricks | Lincoln | Cumberland | Murray | Judith Basin |
| Jefferson | De Kalb | Henry | Logan | Franklin | Nicollet | Lake |
| Kiowa | De Witt | Howard | Marion | Hancock | Nobles | Lewis and Clark |
| Kit Carson | Douglas | Huntington | Marshall | Kennebec | Norman | Liberty |
| Lake | Edgar | Jay | McPherson | Lincoln | Olmsted | Lincoln |
| Larimer | Ford | Jennings | Meade | Oxford | Otter Tail | Madison |
| Las Animas | Fulton | Johnson | Mitchell | Penobscot | Pennington | McCone |
| Lincoln | Greene | Kosciusko | Nemaha | Piscataquis | Pipestone | Meagher |
| Logan | Grundy | Lagrange | Ness | Somerset | Polk | Mineral |
| Mesa | Hancock | Lawrence | Norton | York | Pope | Missoula |
| Moffat | Henderson | Madison | Osborne |  | Ramsey | Park |
| Montezuma | Henry | Marion | Ottawa | MARYLAND | Red Lake | Phillips |
| Montrose | Iroquois | Marshall | Pawnee | Baltimore | Redwood | Pondera |
| Morgan | Jersey | Miami | Phillips | Calvert | Renville | Powder River |
| Otero | Jo Daviess | Monroe | Pottawatomie | Carroll | Rice | Powell |
| Ouray | Kane | Montgomery | Pratt | Frederick | Rock | Prairie |
| Park | Kendall | Noble | Rawlins | Harford | Roseau | Ravalli |
| Phillips | Knox | Orange | Republic | Howard | Scott | Richland |
| Pitkin | La Salle | Putnam | Rice | Montgomery | Sherburne | Roosevelt |
| Prowers | Lee | Randolph | Riley | Washington | Sibley | Rosebud |
| Pueblo | Livingston | Rush | Rooks |  | Stearns | Sanders |
| Rio Blanco | Logan | Scott | Rush | MASS. | Steele | Sheridan |
| San Miguel | Macon | Shelby | Russell | Essex | Stevens | Silver Bow |
| Summit | Marshall | Steuben | Saline | Middlesex | Swift | Stillwater |
| Teller | Mason | St. Joseph | Scott | Worcester | Todd | Teton |
| Washington | McDonough | Tippecanoe | Sheridan |  | Traverse | Toole |
| Weld | McLean | Tipton | Sherman | MICHIGAN | Wabasha | Valley |
| Yuma | Menard | Union | Smith | Branch | Wadena | Wibaux |
|  | Mercer | Vermillion | Stanton | Calhoun | Waseca |  |

Table 425-1 (continued)

List of US Counties with High Radon Potential (Zone 1)

| Yellowstone | NEW JERSEY | Auglaize | Delaware | Miner | Bristol | Marshall |
| --- | --- | --- | --- | --- | --- | --- |
| National Park | Hunterdon | Belmont | Franklin | Minnehaha | Brunswick | Mercer |
|  | Mercer | Butler | Fulton | Moody | Buckingham | Mineral |
| NEBRASKA | Monmouth | Carroll | Huntingdon | Perkins | Buena Vista | Monongalia |
| Adams | Morris | Champaign | Indiana | Potter | Campbell | Monroe |
| Boone | Somerset | Clark | Juniata | Roberts | Chesterfield | Morgan |
| Boyd | Sussex | Clinton | Lackawanna | Sanborn | Clarke | Ohio |
| Burt | Warren | Columbiana | Lancaster | Spink | Clifton Forge | Pendleton |
| Butler |  | Coshocton | Lebanon | Stanley | Covington | Pocahontas |
| Cass | NEW MEXICO | Crawford | Lehigh | Sully | Craig | Preston |
| Cedar | Bernalillo | Darke | Luzerne | Turner | Cumberland | Summers |
| Clay | Colfax | Delaware | Lycoming | Union | Danville | Wetzel |
| Colfax | Mora | Fairfield | Mifflin | Walworth | Dinwiddie |  |
| Cuming | Rio Arriba | Fayette | Monroe | Yankton | Fairfax | WISCONSIN |
| Dakota | San Miguel | Franklin | Montgomery |  | Falls Church | Buffalo |
| Dixon | Santa Fe | Greene | Montour | TENNESSEE | Fluvanna | Crawford |
| Dodge | Taos | Guernsey | Northampton | Anderson | Frederick | Dane |
| Douglas |  | Hamilton | Northumberland | Bedford | Fredericksburg | Dodge |
| Fillmore | NEW YORK | Hancock | Perry | Blount | Giles | Door |
| Franklin | Albany | Hardin | Schuylkill | Bradley | Goochland | Fond du Lac |
| Frontier | Allegany | Harrison | Snyder | Claiborne | Harrisonburg | Grant |
| Furnas | Broome | Holmes | Sullivan | Davidson | Henry | Green |
| Gage | Cattaraugus | Huron | Susquehanna | Giles | Highland | Green Lake |
| Gosper | Cayuga | Jefferson | Tioga | Grainger | Lee | Iowa |
| Greeley | Chautauqua | Knox | Union | Greene | Lexington | Jefferson |
| Hamilton | Chemung | Licking | Venango | Hamblen | Louisa | Lafayette |
| Harlan | Chenango | Logan | Westmoreland | Hancock | Martinsville | Langlade |
| Hayes | Columbia | Madison | Wyoming | Hawkins | Montgomery | Marathon |
| Hitchcock | Cortland | Marion | York | Hickman | Nottoway | Menominee |
| Hurston | Delaware | Mercer |  | Humphreys | Orange | Pepin |
| Jefferson | Dutchess | Miami | RHODE ISLAND | Jackson | Page | Pierce |
| Johnson | Erie | Montgomery | Kent | Jefferson | Patrick | Portage |
| Kearney | Genesee | Morrow | Washington | Knox | Pittsylvania | Richland |
| Knox | Greene | Muskingum |  | Lawrence | Powhatan | Rock |
| Lancaster | Livingston | Perry | S. CAROLINA | Lewis | Pulaski | Shawano |
| Madison | Madison | Pickaway | Greenville | Lincoln | Radford | St. Croix |
| Nance | Onondaga | Pike |  | Loudon | Roanoke | Vernon |
| Nemaha | Ontario | Preble | S. DAKOTA | Marshall | Rockbridge | Walworth |
| Nuckolls | Orange | Richland | Aurora | Maury | Rockingham | Washington |
| Otoe | Otsego | Ross | Beadle | McMinn | Russell | Waukesha |
| Pawnee | Putnam | Seneca | Bon Homme | Meigs | Salem | Waupaca |
| Phelps | Rensselaer | Shelby | Brookings | Monroe | Scott | Wood |
| Pierce | Schoharie | Stark | Brown | Moore | Shenandoah |  |
| Platte | Schuyler | Summit | Brule | Perry | Smyth | WYOMING |
| Polk | Seneca | Tuscarawas | Buffalo | Roane | Spotsylvania | Albany |
| Red Willow | Steuben | Union | Campbell | Rutherford | Stafford | Big Horn |
| Richardson | Sullivan | Van Wert | Charles Mix | Smith | Staunton | Campbell |
| Saline | Tioga | Warren | Clark | Sullivan | Tazewell | Carbon |
| Sarpy | Tompkins | Wayne | Clay | Trousdale | Warren | Converse |
| Saunders | Ulster | Wyandot | Codington | Union | Washington | Crook |
| Seward | Washington |  | Corson | Washington | Waynesboro | Fremont |
| Stanton | Wyoming | PENNSYLVANIA | Davison | Wayne | Winchester | Goshen |
| Thayer | Yates | Adams | Day | Williamson | Wythe | Hot Springs |
| Washington |  | Allegheny | Deuel | Wilson |  | Johnson |
| Wayne | N. CAROLINA | Armstrong | Douglas |  | WASHINGTON | Laramie |
| Webster | Alleghany | Beaver | Edmunds | UTAH | Clark | Lincoln |
| York | Buncombe | Bedford | Faulk | Carbon | Ferry | Natrona |
|  | Cherokee | Berks | Grant | Duchesne | Okanogan | Niobrara |
| NEVADA | Henderson | Blair | Hamlin | Grand | Pend Oreille | Park |
| Carson City | Mitchell | Bradford | Hand | Piute | Skamania | Sheridan |
| Douglas | Rockingham | Bucks | Hanson | Sanpete | Spokane | Sublette |
| Eureka | Transylvania | Butler | Hughes | Sevier | Stevens | Sweetwater |
| Lander | Watauga | Cameron | Hutchinson | Uintah |  | Teton |
| Lincoln |  | Carbon | Hyde |  | W. VIRGINIA | Uinta |
| Lyon | N. DAKOTA | Centre | Jerauld | VIRGINIA | Berkeley | Washakie |
| Mineral | All Counties | Chester | Kingsbury | Alleghany | Brooke |  |
| Pershing |  | Clarion | Lake | Amelia | Grant |  |
| White Pine | OHIO | Clearfield | Lincoln | Appomattox | Greenbrier |  |
|  | Adams | Clinton | Lyman | Augusta | Hampshire |  |
| NEW HAMPSHIRE | Allen | Columbia | Marshall | Bath | Hancock |  |
| Carroll | Ashland | Cumberland | McCook | Bland | Hardy |  |
|  |  | Dauphin | McPherson | Botetourt | Jefferson |  |